Proactive Wearable Risk Monitoring

An Patient Escalation Model for Consumer Wearables

Architecture & Design Reference  ·  v1  ·  February 2026

“This architecture explores a risk monitoring and escalation pathway that incorporates consumer wearables or other monitoring devices - a path toward a more proactive healthcare model.”

| Version v1 | Date February 2026 | Status Conceptual / Pre-prototype | License Apache 2.0 |
| --- | --- | --- | --- |

# 1. Motivation

Consumer wearables and health monitoring devices record a wealth of patient health information. With the right risk scoring and context, the information gathered from consumer wearables could be used to monitor for negative health episodes, allowing risks to be flagged proactively rather than waiting for a patient to report symptoms. This is especially powerful for patients with severe conditions.

Core Hypothesis Proactive risk monitoring and escalation, informed by EHR context and consumer wearables will improve patient outcomes by allowing critical episodes to be caught earlier, reducing hospitalization and cost.

## Design Principles

- Transparency over complexity

- Modular modifier layers

- Clear separation of severity and confidence

- Governance-ready architecture

## Relationship to Existing Approaches

| Approach | What It Does Well | What This Architecture Adds |
| --- | --- | --- |
| Patient Symptom Reports | Allows patients to escalate symptom concerns. In some cases, proactively reaches out to patients | Wearables allow targeted and timely questioning or escalation to providers without relying on patient symptom reports if vitals are critical risk. |
| Provider Follow Up | Provider directly involved for targeted symptom questions. | Proactively involves the provider if critical risk is present. Could catch episodes standard follow ups may not due to timing. |
| Consumer Wearable dashboards | Allows providers to view and monitor patient vitals. | Dashboards would provide a large amount of information that requires provider review and decision making. This narrows provider attention to the most relevant and actionable cases. |

# 2. Full System Architecture

The complete architecture for v1 consists of 6 distinct components that are defined in the Layer Overview. For all thresholds defined, in production use cases thresholds would be defined using clinical guidelines, validated by medical governance, and should be version controlled.

## Layer Overview

The architecture has six named components that ingest information from the EHR and consumer wearables and output a patient risk flag (medium-high) or a provider risk flag (critical).

| Component | Description | Key Points |
| --- | --- | --- |
| Wearable input layer | API call to wearable device to get available patient vitals (HR, HRV, respiration, ect.) (See section 3) | - API call - Vital ingestion |
| Wearable Risk Engine | Dual-engine system. Calculates acute risk and chronic risk of ingested vitals based on vital severity. (See section 4) | - Wearable risk score - Severity Score - Context Modifier |
| EHR Modifier Layer (recommended, not required) | Modifies the risk score based on relevant risk factors present and ingestible from the EHR. (See section 5) | - Diagnosis modifier - EHR API call |
| Confidence Modifier | Modifies the severity score by confidence modifier (See section 6). | - Confidence modifier |
| Risk Calculation | A finalized risk score is calculated by the wearable risk score, confidence score, trend modifier, and Diagnosis modifier (See section 7) | - Risk Score Calculation - Confidence score - Trend Modifier |
| Risk Flag | Risk score determines if a risk flag should be set. If risk is Medium, the risk flag is set to patient follow up. If risk is high, the risk score is set to Nurse triage. If risk is critical, risk score is set to provider escalation. EHR API is called to set the risk flag. EHR rules used to set follow up actions (See section 8). | - Risk flag - Patient follow up - Provider escalation - EHR API call sets flag - EHR configuration |
| Risk Dashboard | Dashboard displays patient risk scores and flag decisions with auditable reasoning for risk scores (See section 9). | - Dashboard with risk reasoning |

Risk Formula Stack

## 3. Wearable Input Layer

Consumer wearables record several vitals that can provide insight into a user’s current health status. For V1, the focus will be on vitals that are relatively accurate, heart rate (HR), resting heart rate (RHR), predicted max heart rate, heart rate variability (HRV), step count, atrial fibrillation (AF), age. (See section 12 for open problems and considerations).

Consumer wearable data will be ingested via an API call. V1 simulates this with a mock API and consumer wearable data. For the purposes of V1, mock API calls should be completed every hour However, in a production use case, the API call would need to be adjusted to work for a variety of consumer wearables and frequency of call should be further scrutinized.

## 4. Wearable Risk Engines

The wearable risk engine calculates the pre-modifier risk score for the combination of vitals that is provided by the consumer wearable. V1 will implement a rule based risk scoring system. (See section 13 for future ideas).

## 4.1 Acute Danger Engine

The Acute Danger Engine calculates vitals to determine if there is potential acute clinical risk for a patient. The Acute Danger Engine flags fast decay, is intended to prevent catastrophic events, and has a higher sensitivity. Scoring for the Acute Danger Engine is based on population safety thresholds.

### Acute Severity Score

This engine will calculate a severity score for each vital based on the severity of vitals (0-100 scale). In V1, vitals calculated in the acute severity score are heart rate (HR) and atrial fibrillation (AF).

The below calculation will be used to determine the acute time adjusted severity score where the acute_base is the vital in comparison to the population threshold, short_persistence_factor accounts for the persistence of the vital over the monitoring period (1 hour), and activity_modifier is a modifier based on the activity level recorded.

Acute_time_adjusted = acute_base * short_persistence_factor * activity_modifier

For V1, a population safety baseline will be used to determine absolute threshold score. Baseline and scoring will be as follows. This assumes a population of non-athlete adults. No additional demographic factors are accounted for in V1:

| Heart Rate (severity) | Population Threshold Score | Interpretation |
| --- | --- | --- |
| 50–100 | 0 | Normal adult resting/activity range |
| 100–110 | 20 | Mild tachycardia |
| 110–120 | 40 | Moderate tachycardia |
| 120–140 | 65 | Significant tachycardia |
| >140 | 85 | Severe tachycardia — high acute concern |
| 40–50 | 40 | Mild bradycardia |
| 30–40 | 75 | Significant bradycardia |
| <30 | 95 | Critical bradycardia |
| >180 | 95 | Extreme tachycardia (critical) |

| Vital duration (HR) | Persistence Factor |
| --- | --- |
| < 5 minutes | .75 |
| 5 -15 minutes | 1 |
| 15 - 30 minutes | 1.25 |
| 30+ | 1.5 |

| Atrial Fibrillation (severity) | Population Threshold Score | Interpretation |
| --- | --- | --- |
| No AF detected | 0 | No artifact |
| Single brief detection (<1 min) | 40 | Possible artifact |
| 2+ detections within 30 min | 60 | Concerning |
| Sustained AF >5 min | 75 | High concern |
| Sustained AF >15 min | 85 | Very concerning |
| AF + HR >140 | 95 | Hemodynamic risk |
| AF + brady <40 | 95 | Critical instability |

Below are the activity modifiers accounted for in V1. If max HR is not recorded, max_HR = 202 - age (in years).

| Recorded Activity | Modifier |
| --- | --- |
| HR >85% of predicted max HR AND 150 > steps/min >100 | .75 |
| HR >85% of predicted max HR AND > 150 steps/min | .5 |
| HR > 85% predicted max AND <50 steps per minute | 1.5 |
| HR > 170 for +20 minutes AND steps <100 | 1.5 |

The below conditions are considered severe but do not fit into the above calculations. They will result in a score of 100 regardless of above scoring.

- HR >200 AND steps <50 AND duration >10

- HR >200 AND duration >15 min regardless of steps

- HR >200 AND any AF detected

- HR < 40

## 4.2 Chronic Decompensation Engine

The Chronic Decompensation Engine tracks how vitals over time are trending to escalate negative health trends. The Chronic Decompensation Engine is intended for slower moving deterioration detection. The engine uses data from resting periods where steps/min < 10 or the user is in a sleep state and no acute alert is active.

This engine requires 14 days of data to establish a median RHR. The engine will then calculate the following:

ΔHR = Current 7-day resting mean – Baseline

Slope of RHR

%ΔHRV = (Current 7-day resting mean) – Baseline / Baseline

| Change in RHR Activity | Severity Score |
| --- | --- |
| ΔHR < 3 bpm | 0 |
| ΔHR 3–5 bpm | 25 |
| ΔHR 5–8 bpm | 45 |
| ΔHR >8 bpm | 65 |
| ΔHR >10 bpm | 80 |

Add +10 for consistently upward trending RHR if R² of regression > 0.4 and persistence ≥ 5 days.

| Change in HRV from Baseline | Severity Score |
| --- | --- |
| <5% | 0 |
| 5–15% | 30 |
| >15% | 60 |

Individual severity scores will be calculated into a chronic severity score as a weighted sum. A persistence modifier will also be applied.

Chronic_time_adjusted = ((0.55 × Resting HR Drift) + (0.45 × HRV Degradation)) * (persistence modifier)

| Days of Sustained Irregular Recordings | Persistence Factor |
| --- | --- |
| < 5 Days | .75 |
| 5 - 6 Days | 1 |
| 7 - 14 Days | 1.25 |
| +14 Days | 1.5 |

The below conditions are considered severe but do not fit into the above calculations. They will result in a score of 100 regardless of above scoring.

- ΔHR >8 bpm for >7 days

- %ΔHRV >20% for > 10 days

- R² of regression > 0.8 for > 5 days

## 6. EHR Modifier Layer (recommended, not required)

Context contained in the patient chart is critical to understanding patient vitals. For instance, a high heart rate for a patient without any diagnoses should have a different escalation score than a patient diagnosed with congestive heart failure.

The EHR Modifier layer should be an optional layer that allows for an EHR API to be called to ingest specific relevant patient fields. For V1, we will limit these fields to patient problem list (diagnoses). Focus diagnoses for V1 will be generalized anxiety, congestive heart failure, and history of arrhythmia, and the mock API will be called daily. In a production environment, the API call frequency should be scrutinized and APIs will be EHR dependent.

For each diagnosis in scope, the EHR modifier should give a defined value (0-1.0) that will be used to adjust the vital risk score.

See section 12 for further discussions on open problems with diagnosis handling and EHR reliability.

Diagnosis Modifiers for V1:

## Acute DX Modifier Table

| Diagnosis | Multiplier |
| --- | --- |
| None (baseline) | 1.0 |
| CHF | 1.25 |
| AF history | 1.15 |
| Anxiety | 0.85 |

## Guardrails for anxiety: HR > 190 → force ≥90 score; HR < 35 → force ≥90; AF + HR >160 sustained → force ≥90

Guardrails for CHF: If patient has sustained AF > 15 or 2+ AF, force 100

## Chronic DX Modifier Table

| Diagnosis | Global Multiplier |
| --- | --- |
| None | 1.0 |
| CHF | 1.2 |
| AF history | 1.1 |
| Anxiety | 0.9 |

Final_chronic = (chronic_time_adjusted) * (max(dx_multiplier))

Final_acute = (acute_time_adjusted) * (max(dx_multiplier))

## 6. Confidence Layer

The Confidence Modifier adjusts the risk score based on confidence in the data being used in scoring. The following equation will be used to calculate the Confidence Modifier:

Confidence (capped at 100) = (coverage_ratio * 0.6) + (sample_completeness * 0.3) + (recency_factor * 0.1)*(1.2)

Where:

coverage_ratio = % of lookback window with usable data
sample_completeness = expected samples vs received
recency_factor = scaled based on time since last reading

Both the acute_time_adjusted and chronic_time_adjusted are multiplied by the confidence modifier.

If vitals are extreme:

- HR >200 AND steps <50 AND duration >10

- HR >200 AND duration >15 min regardless of steps

- HR >200 AND any AF detected

- HR < 40

- Sustained AF > 15 min or 2+ AF with CHF

- ΔHR >8 bpm for >7 days

- %ΔHRV >20% for > 10 days

- R² of regression > 0.8 for > 5 days

And confidence >.2, the lowest flag able to fire is RED. Critical is possible if calculated as such. If confidence is below .2, the flag will return “Insufficient data”.

Similarly, if the following conditions are met and confidence is >.2, the lowest flag able to fire is YELLOW.

- HR >180 AND steps <50 AND duration >10

- HR >180 AND duration >15 min regardless of steps

- HR >180 AND any AF detected

- HR < 45

- Sustained AF > 5 min or 2+ AF with CHF

- ΔHR >5 bpm for >5 days

- %ΔHRV >15% for > 7 days

- R² of regression > 0.6 for > 5 days

## 7. Risk Calculation

The outputs from the previous 2 sections, risk score and confidence modifier, are calculated with the following formula to determine a risk flag:

acute_risk (capped at 100)= final_acute * confidence

Chronic_risk (capped at 100) = final_chronic * confidence

## 8. Escalation Policy

Once a risk score has been calculated, a flag for risk can be set for the patient. The flag will be set as soon as calculation is completed. For V1, the following flag structure will be used:

| Condition | Flag | Behavior |
| --- | --- | --- |
| None elevated | Green | Vitals are good and/or confidence is low. No action needed, no follow up. |
| chronic_risk 60–80 or acute_risk 60-70 | Yellow | Vitals are out of range, conditions may factor in, confidence is medium to high. Patients are flagged with intelligent follow up questions if integration is available. Flag patient as potential risk on dashboard. |
| chronic_risk > 80 or acute_risk 70–85 or (acute risk > 85 & confidence <0.6) | Red | Vitals are severely out of range, conditions may factor in, confidence high. Nurse flagged to follow up with patient if integration available. Flag patient as risk on dashboard |
| acute_risk > 85 & confidence > 0.6 | Critical | Vitals are severely out of range, conditions are present, confidence is high. If integration available, alert provider immediately. Patient needs medical care. Dashboard flags patient as critical risk on dashboard |
| Confidence < .2 | Insufficient Data (no cooldown) | Confidence is too low to extrapolate data. Appears on dashboard for auditability. |

Additional rules for escalation: 24-hour cooldown for same/lower flag & active case suppression - if there is an active case for the patient, no escalations will be given.

For patient questions, the questions sent to patients and settings to determine if these should be automatically sent are configurable. Notification of patient response and / or lack of response is also configurable. For V1, we will triage patient responses with pre-defined escalation logic for questions and a lack of patient response will not be escalated.

## 9. Risk Dashboard

The risk flags will also feed into a patient dashboard that displays the risk flag status and reasoning behind status. Statuses should allow auditability into the system decisions and a drilldown view with raw data and risk scoring calculations should be available.

The dashboard should also include a log of rule triggers per calculation and the rule set version used for calculation scoring.

Critical Design Principle The system must be auditable and decision reasoning must be clear. The dashboard is necessary to surface decision reasoning.

## 10. Governance Considerations

Raw data will be stored in a database for 30 days to ensure auditability. Summarized data (averages, medians, and risk calculations) will also be stored for 1 year. In a production environment, data should be stored in compliance with HIPAA and securely stored.

Metrics should be tracked to determine success of the model. For false positives, an ignore button on escalations should be implemented and tracked. Additionally, unacted upon non-green flags should be tracked in a “no action” metric. False negatives are more difficult to track. For this, evaluation of critical events for patients using this model will need to be tracked.

# 11. Compliance and Security

This framework is operating under the assumption that only mock data is to be used. Given the sensitive nature of the data that would be used in a production model, additional compliance and security measures would be needed to support this. Below are some additional requirements, however, this is a non-exhaustive list.

- Patient dashboard would need security access dependent on provider / patient relationship with authentication for access.

- Integrations would need to be secure and HIPAA compliant

- Integrations would require BAAs

- Patient consent would need to be gathered and documented to provide both EHR access and clinician access to wearable data.

- A balance between auditability and minimal patient data storage would need to be addressed.

# 12. Open Problems

This architecture is practical with current infrastructure in its core components. The following problems are documented honestly, not as failures but as known design challenges requiring resolution before production deployment.

| Problem | Status and Mitigation |
| --- | --- |
| EHR problem lists are often not well maintained | Often diagnoses are not resolved on problem lists which could lead to additional scoring that should not be relevant for the patient. This is currently unresolved, but would need to be considered before a production implementation. |
| Wearables are not equal in accuracy | Consumer wearables are not always accurate and not every consumer wearable is equal in accuracy. While we could make adjustments to vital confidence per wearable model, this would be imperfect. The current architecture does not address this issue |
| Risk Scoring requires more research | The risk scoring in V1 is simplified and does not capture the full complexity of wearable monitoring. Scoring will need to be tuned in V1 to account for edge cases and daily human activity. This is an active area of improvement. |
| Diagnosis scoring requires clinical expertise | Each diagnosis needs an individual score that accurately represents how it impacts the interpretation of the diagnosis. This would require both clinical input and tuning for all supported diagnoses. |

# 13. Ideas for Future Versions

This is a list of ideas that may be explored in future versions. These ideas are out of scope for the purposes of V1, but would improve the architecture.

| Idea (Futured) | Version Recommendation |
| --- | --- |
| Use machine learning or LLMs to tune severity scoring based on previous patient escalations and events. | V3+ |
| Add additional demographic modifiers to the population baseline. | V2 |
| Add recovery rate to chronic decomposition engine | V2 |
| Add z score modifier to account for baseline deviations in acute risk engine as a secondary signal | V2+ |
| Include medication data as a modifier | V2+ |
| Include hospitalization as a modifier | V2 |
| Design additional routes for addressing lack of patient response for yellow flag | V2 |

February 2026 · v1

Licensed under MIT. This document may be freely shared, cited, and extended.